(State Private University Established Under the Shiv Nadar University Act, 2018)

School of Engineering

Department of Computer Science and Engineering

Regulation 2021

CS1702 Introduction to Internet of Things & Lab

II sem - B.Tech CSE (IoT) - A section

Electricity Energy Monitoring With Arduino

MINI PROJECT ABSTRACT

Student Members

Name 1: Aditi Kannan         Reg No: 21011102010

Name 2: Ahana Chatterjee   Reg No: 21011102011

Name 3: Aishwarya R          Reg No: 21011102012

Name 4: Kruti Dasari           Reg No: 21011102025

April 2022

ABSTRACT

- Objective

Efficient energy utilization plays a very vital role for the development of smart grid in power system. So, proper monitoring and controlling of energy consumption is a chief priority of the smart grid. The existing energy meter system has many problems associated to it and one of the key problem is there is no full duplex communication. To solve this problem, a smart energy meter working example is proposed based on Internet of Things (loT). The proposed energy meter example controls and calculates the energy consumption using ESP32 board, a Wi-Fi module and uploads it to the cloud from where the consumer or producer can view the reading. Therefore, energy analyzation by the consumer becomes much easier and controllable.  Thus, this smart meter helps in home automation using IoT and enabling wireless communication which is a great step towards Digital India.

The IoT based smart energy meter is based on Arduino. In this system we reduce the human participation in electrical energy maintenance. The electricity bills are issued only when they will get an idea about their consumption. Bill is distributed only once in a month in India. So the consumer will be in dark about their energy usage during this period of time. This procedure has to be repeated numerous times to efficiently control the energy usage in a month. If customer can check their leap in the area of energy consumption on their mobile phone or laptop instead of checking energy meter, it will be a great leap in the area of energy management. Since nearly all of the people are today 24*7 online, it will really a benefit if they can check their energy consumption online from everywhere on the globe.

- Methodology

In this project we make a Smart Electricity Energy meter using Arduino and ESP8266 Wi-Fi module which can not only sends you a SMS/Email of your electricity Bill but also you can monitor the energy uses anytime and from anywhere in the world. Here we have used a Current Sensor ACS712 to measure the energy consumption.

- Working Principle

Measuring current especially AC current is always a tough task due to the noise coupled with it improper isolation problem etc. But, with the help of this ACS712 module which was engineered by Allegro thing have become a lot easier. This module works on the principle of Hall-effect, which was discovered by Dr. Edwin Hall. According his principle, when a current carrying conductor is placed into a magnetic field, a voltage is generated across its edges perpendicular to the directions of both the current and the magnetic field. The major advantage of using ACS712 Current Sensor is that is can measure both AC and DC current and it also provides isolation between the Load (AC/DC load) and Measuring Unit .

We will be using the Analog pin of Arduino to read the output voltage (Volt) of the module, which will be 512(2500mV) when there is no current flowing through the wire. This value will reduce as the current flows in negative direction and will increase as the current flows in positive direction.

- Procedure

Connect Receiving end of ESP12 to transmitting end of Arduino

Connect Transmitting end of ESP12 to the receiving end of arduino

There is one analog pin available in NodeMCU (ESP12), we could use that pin but ESP series can take upto 3.3 volts on their pins. As we are using current sensor which can give upto 5 Volts so, it can damage our Wi-Fi module that’s why we are not using standalone NodeMCU.

To make output of current sensor 3.3V instead of 5V, we cannot use voltage divider circuit between Current sensor and analog pin of NodeMCU because as we discussed above about the current sensor that at 2.5Volts output, current is 0Amp.

So, Arduino will read the current sensor value through analog pin and send it to the Wi-Fi module ESP12 using Serial communication. Use voltage divider circuit at receiver pin of NodeMCU so that receiver pin can get upto 3.3 Voltage level.

- Components Required

- Arduino Uno board

- ESP12/ NodeMCU – for wifi connection

- ACS712-30Amp Current sensor

- Any AC Appliance

- Male-Female Wires

- Conclusion

# Energy Monitoring using IOT is a very innovative application of the internet of things which plays a vital role in upcoming years. It is used to control home appliances remotely over the cloud from anywhere in the world. In the proposed project current sensor is used to sense the current and display it on the web using IoT. The system updates the information every 4 to 3 seconds because it has some delay due to the relay module connecting with the appliances to make the appliances home automated.

# In this new system, load consumption of the appliances is accessed using Wi-Fi technology and it will help consumers to avoid unwanted use of electricity. An IoT system where a consumer can monitor energy consumption and pay the bill Online. Also, a system where a user can receive SMS, when he/she crosses the threshold of electricity usage slab can be equipped. We may further enhance this system where a consumer receives text message whenever someone is trying to steal the electricity within a 100m of area but that is not discussed in this model. Also using cloud analytics we will predict future energy consumptions. This system can also perform home automation which is another advantage of making this project.